# Software Documentation

# 1. Software Overview

The software controls the complete functioning of the LieDet system. It reads physiological signals from sensors, processes them in real time, and outputs results through an OLED display and a mobile application.

The ESP32 collects data from the MAX30102 (heart rate) and GSR sensor, processes the signals to calculate stress levels, and determines a response classification. The processed data is displayed on the OLED screen and transmitted to a mobile app via Bluetooth.

Platforms used:
- MicroPython (ESP32) – sensor reading and processing
- MIT App Inventor – mobile app interface and visualization
- Bluetooth Low Energy (BLE) – communication

# 2. System Logic

1. User wears the device and places finger on sensor
2. ESP32 reads data from MAX30102 and GSR
3. System performs calibration to set baseline
4. Sensor data is continuously collected and filtered
5. BPM, GSR, amplitude and HRV are calculated
6. Data is compared with baseline
7. Stress score (0–100) is generated
8. Score is classified (Calm / Elevated / High)
9. If question is triggered, response window is monitored
10. Verdict is generated (True / Uncertain / Lie-like)
11. Data displayed on OLED
12. Data sent via BLE to app
13. App displays and logs data

# 3. Flowchart

Start → Initialize → Calibration → Read Sensors → Process Data → Calculate Score → Check Question Mode → Generate Output → Send via BLE → Display → Loop

# 4. App Flow (MIT App Inventor)

Screen Flow:
Main screen → Connect → Display data → Log history

Button Actions:
- Connect button connects to ESP32 via BLE
- Clear button clears log data

Data Flow:
ESP32 sends formatted string (e.g., BPM:78|GSR:1400|SCORE:45)
App receives using StringsReceived
Data is displayed and added to log