# Fabrication and Integration Process

### 1. Material Choice Justification :

Initially, we aimed to create a fully integrated and wearable design using 3D-printed components. A custom finger clip for the MAX30102 oximeter was designed and printed to ensure stable and consistent contact with the user’s finger. Additionally, a 3D-printed wrist band was developed to house the ESP32, sensors, and display, with the goal of achieving a compact and aesthetically clean wearable form.

However, during testing, these designs presented practical challenges. The oximeter clip did not accommodate the required breadboard and wiring, which were essential for stable electrical connections during prototyping. This made the clip bulky and reduced comfort and usability. Similarly, the 3D-printed band lacked flexibility and adjustability, making it less suitable for different hand sizes and prolonged wear.

As a result, the design was adapted to prioritize functionality and reliability. A Velcro-based adjustable band was chosen for its flexibility, ease of use, and better fit across users. For structural support, a cardboard backing was used to mount the ESP32, sensors, and breadboard securely. While less refined in appearance, this solution provided improved stability, faster iteration, and easier modifications during development.

This iterative shift reflects a practical engineering trade-off: moving from an ideal, fully integrated design to a more robust and testable prototype.

2. Fabrication Steps :

  Initial Design Phase

- Designed a 3D-printed finger clip for the MAX30102 sensor to maintain consistent finger placement

- Designed a 3D-printed wrist band to house all components

  3D Printing and Testing

- Printed the oximeter clip and wrist band

- Tested fit, comfort, and sensor stability

- Identified issues with space constraints, wiring complexity, and user comfort

  Design Iteration

- Evaluated limitations of the printed components, especially with breadboard integration

- Decided to prioritize modularity and ease of assembly over compactness

  Final Fabrication

- Mounted ESP32, sensors, and breadboard onto a cardboard base

- Secured the setup using an adjustable Velcro strap for wearability

- Positioned the MAX30102 sensor for stable finger contact

- Integrated the OLED display for visibility

  Integration and Testing

- Connected all components via I2C and analog inputs

- Verified sensor readings and display output

- Tested comfort, stability, and real-time performance during use